SCHEDULE OF DIMENSION AND AREA REGULATIONS

D	Maximum		Minimum lot		Maximum 		Minimum	Minimum	Minimum	Minimum

I	Height of 		Area when		Percentage of lot	Lot Width	Front		Side		Rear

S	Buildings		Served by		area which may	at front		Yard		Yard		Yard

T				Sanitary		be covered by		yard line

R				Sewer			all buildings

I

C

T       (Feet)			(Sq.Ft.)		(%)			(Feet)		(Feet)		   (Feet)	   (Feet)	______

R-1	30’ or 			8,000			    35		   	70		   25		    10		      20

2 stories												Accessory buildings

shall be 8’ from all lot

lines and all other buildings

__________________________________________________________________________________________________________________

R-2	30’ (1) or		6,500			    35			Single		   25		    10		     15

Stories			Minimum area for multiple-			Family				Accessory Buildings shall

Family dwellings shall be 			60				be 5’ from all lot lines and

R-3	45’ (1) or		the single-family minimum			Multiple			 all other buildings.

3 stories		plus 35% of that minimum			Family

for each additional dwelling			75

unit

__________________________________________________________________________________________________________________

CR(5)	40’ or			NA			    65			50		    40 or (5)	30’(2)		      15’ or

HC	4 stories										    60				    30’(2)

__________________________________________________________________________________________________________________

CC	50’ or			NA			    NA(5)		NA	       - 		    - 		           -

5 stories

__________________________________________________________________________________________________________________

I-1	 - 			NA			    65			100		75’(4)		25’(4)		    50’(4)

___________________________________________________________________________________________________________________________________________________________________________

- Height of building may be increased up to 50’ provided each side yard is increased by the same amount over the required yard

Minimum that the building height is increased over the 30’ maximum height.

- Only applies to side adjacent to a residential zoning district.

- No industrial building shall be located closer than 100’ to a residential district

- Note parking regulations.

- 40’ allowed where sanitary sewers are available; 60’ required where sanitary sewers are not available

- If the proposed use is commercial, HC dimensions shall be followed, if the proposed use is multi-family , R-3 dimensions shall control